Tract 3 (William D. Brown, Owner)

(Survey dated February 17, 2025)

All that tract or parcel of land lying and being in Land Lot 691 of the 19th District, 2nd Section, Cobb County, Georgia, and being more particularly described as follows:

TO FIND THE TRUE POINT OF BEGINNING, COMMENCE at a 3/4" open top pipe found at the corner common to Land Lots 642, 643, 690, and 691;  thence along the line common to Land Lots 642 and 691 N88°29’13”E for a distance of 44.89 feet to a 1/2" rebar found at the intersection of said Land Lot line and the westerly right of way of Ernest Barrett Parkway (a.k.a. East-West Connector, a.k.a. West Cobb Loop) (a 109 foot public right of way and limited access highway); thence along said westerly right of way the following courses and distances:

264.52 feet along the arc of a curve to the right, said curve having a radius of 6820.99 feet and being subtended by a chord of S11°04’32”E, 264.50 feet to a concrete monument found;

S09°57’51”E for a distance of 523.11 feet to a 1/2" rebar found on the westerly right of way of Ernest Barrett Parkway, said point being the TRUE POINT OF BEGINNING.

FROM THE TRUE POINT OF BEGINNING AS THUS ESTABLISHED; thence continuing along said westerly right of way S09°58’05”E for a distance of 529.06 feet to a 1/2” rebar with cap set at the intersection of said westerly right of way and the line common to Land Lots 691 and 716; thence leaving said right of way and along said common Land Lot line S89°20’57”W for a distance of 101.08 feet to a 3/4" crimped top pipe found; thence leaving said common Land Lot line N01°02’27”E for a distance of 522.31 feet to a 1/2" rebar found on the westerly right of way of Ernest Barrett Parkway, said point being the TRUE POINT OF BEGINNING.

Said tract or parcel of land contains 0.606 acres.