Sec. 8-76. Street lighting.

(a)	Required. Any person or entity developing a new subdivision project constructing new streets to be dedicated to the City of Powder Springs or utilizing existing city or county roadways (or combination thereof) shall be required to provide streetlights which conform to the standards included in this section. In order to ensure adequate illumination of public rights-of-way and promote safety and security, the American National Standard Practice for Roadway Lighting of the Illuminating Engineering Society, as approved by the American National Standards Institute (1983), and as from time to time amended, is hereby adopted as the standard for the installation and operation of lighting in the city, with the following exceptions:

(1)	Lighting fixtures installed within the public rights-of-way to be operated for the purpose of street illumination shall comply with the following standards. The minimum average horizontal footcandle illumination (AHFI) level by roadway classification shall be:

| Roadway Classification | Commercial Area (AHFI) | Intermediate Area (AHFI) | Residential Area (AHFI) |
| --- | --- | --- | --- |
| Major | 1.2 | 0.9 | 0.6 |
| Collector | 0.8 | 0.6 | 0.4 |
| Local or residential | 0.6 | 0.5 | 0.3 |

The uniformity of illumination shall be such that the point of lowest illumination shall have at least one-third of the average horizontal footcandle required illumination level, except that on local or residential streets it may be not less than one-sixth of this average. The fixtures shall be mounted a minimum of 25 feet above the ground, and each fixture shall have appropriate arm length to illuminate the street.

(b)	Plan. The subdivider or developer shall submit a street light layout prepared by the utility company which will provide the lighting service showing the exact location of street lights within the subdivision. The lighting layout must be approved by the director of infrastructure and development, or his/her designee.

(c)	Costs. The subdivider or developer must pay for all costs to the power company for pole fixtures, and other related items or materials necessary for the installation. Evidence of such completed payment shall be provided to the community development director at the time of final plat submittal. The subdivider or developer shall bear the costs for all monthly street light charges until all residential structures have been completed. At such time of completion, the subdivider or developer shall provide written notification to the City of Powder Springs (via its community development director) requesting that the billing be transferred to the City of Powder Springs. Any such request for transfer of billing shall be presented to and approved by the mayor and council as part of the final plat approval.

(Ord. No. 2023-014, § 1(Exh. A), 9-18-2023)

Sec. 15-77. Process for review and approval of a final plat.

Upon completion of all requirements, the subdivider or property owner can file an application for final plat approval. The procedures for final plat review and action shall be as specified in this section.

(a)Review for completeness and application acceptance. The community development department shall review the final plat application for completeness within no more than 10 calendar work days from the time of submission. Incomplete applications will not be processed and will be returned to the applicant.

(b)Distribution and agency review of final plat. The community development department shall forward a copy of the final plat application to other city departments as may be appropriate, the Georgia Department of Transportation if the proposed subdivision has frontage on or proposes access to a state or federal road, or others as appropriate, for their review and comment. Agency review shall specifically include the approval from the Cobb County Health Department if septic tanks are proposed and public utility providers in cases where connection to public water and/or sewer is proposed or required.

(c)Time period for completion of review. Except for final plats that have frontage on or propose access to a state or federal road which require review by the Georgia Department of Transportation (which require a 30 day review period, within no more than 25 calendar work days following receipt of a complete final plat application, during which agency review shall be completed, the community development department shall indicate on the final plat or in writing all comments related to compliance with this article and the development code. The department shall provide all comments to the applicant for resolution, who shall work with each department as necessary to resolve all issues.

(d)Action. When the department of community development has determined that the final subdivision plat is in compliance with the approved preliminary plat for the subdivision (if applicable) and the requirements of this development code and consistent with the comprehensive plan, it shall be approved. The owner shall be responsible for compliance with all codes, regulations, and zoning requirements and for the satisfaction of all the noted and written comments. If it is determined that the final plat is not in compliance with the requirements of this development code, the department shall disapprove the final plat. Action must be taken by the department to approve, conditionally approve, or disapprove the final plat application within no more than 60 days from receipt of a completed application.

(e)Variances. The community development department shall not approve any final plat that would result in a lot or situation that would clearly require a variance, until or unless such variance is lawfully obtained. An application for variance submitted after submission of the final plat application will extend the time frame for action as specified in this section.

(f)Assignment of addresses. If the final plat is approved, the community development director shall ensure that an address is assigned to each lot in the subdivision.

(g)Certificate of final plat approval. Upon approval of the final plat, the community development director or his or her designee and, if public improvements are to be dedicated the Mayor of Powder Springs, shall sign and date the certificate of final plat approval stamped or printed on a reproducible copy of the final subdivision plat. One copy of the approved drawing shall be transmitted to the applicant and the community development department shall retain additional copies for distribution to the department of public works and other agencies as may be appropriate.

(h) Transfer of streetlights. If the final plat is approved, the community development director shall ensure that the streetlights have been fully paid by the developer and the billing has been transferred to the City.