Ord 2023-014

Exhibit A

Contents

Article 8, Land Development Requirements.	2

ROAD ACCEPTANCE AND BONDING PROCESS	2

Article 8, Land Development Requirements.	5

STREET LIGHTING	5

RENTAL CAP	6

Article 2, Table of Permitted and Special Uses in Mixed-Use and Non-Residential Zoning Districts.	7

HOME OCCUPATIONS IN CBD AND MXU	7

PACKAGE STORE IN NRC	7

Article 3, Specific Use Provisions

FUEL PUMPS………………………………………………………………………………………………………………………………………………………………..8

# Article 8, Land Development Requirements.

## ROAD ACCEPTANCE AND BONDING PROCESS

Table 8-1

Minimum Required Street Right-of-Way and Pavement Widths (Excludes State and U.S. Numbered Highways)

| Street Classification | Minimum Required Right-of-Way Width* | Minimum Required Pavement Width | Minimum Required Sidewalk Width |
| --- | --- | --- | --- |
| Arterial | 100 feet | 52 feet with 13 foot lanes | 5 feet |
| Major Collector | 80 feet | 52 feet with 13 foot lanes | 5 feet |
| Minor Collector | 60 feet | 28 feet with 14-foot lanes | 5 feet |
| Minor Collector – Residential | 60 feet | 24 feet | 5 feet |
| Local Commercial or Industrial Street | 60 feet | 28 feet with 14-foot lanes | 5 feet |
| Local Residential Street | 50 feet | 20 feet; 18 feet with vertical curb and gutter | 5 feet |
| Alley | 20 feet | 16 feet within 20 foot roadbed | 5 feet |

* Additional 5 feet of right-of-way required on each side where the Comprehensive Plan envisions future trails.

In residential subdivisions or land developments, streets that serve 100 dwelling units or more shall be improved to or exceed “minor collector” street standards, while those serving fewer than 100 dwelling units shall be improved to or exceed “local” street standards.

Sec. 8-64. Dedication of Right-of-Way for Existing Substandard Streets.

Subdivisions or developments that adjoin an existing street that does not meet the right-of-way width required by this development code for said street as classified, shall dedicate up to one-half of the right-of-way needed to meet the minimum right-of-way width required for the street along the entire property frontage. If any part of a subdivision or land development fronts on both sides of an existing street, all of the right-of-way needed for the existing street to meet the right-of- way standards of this development code for the street shall be dedicated along the entire property frontage.

Sec. 8-65. Improvement of Existing Streets.

- Existing streets that adjoin a major subdivision or land development shall be improved to the minimum standards of this development code from the centerline of the street along the development’s or major subdivision’s frontage. Such required improvements specifically include widening of the street from the centerline to the pavement width specified for the street as classified, and the installation of curb and gutter and, if required, sidewalk, along the entire property frontage abutting the existing street.

- Proposed subdivisions or nonresidential developments accessing an existing city street that has less than 16 feet of pavement width shall be required to upgrade the city street to 18 feet of pavement, with 2 2-foot shoulders, adequate drainage ditches (if required), and obtain the necessary right-of-way to accomplish these improvements. The city street shall be improved by the subdivider or developer the length of the road required to reach a city street or county road that meets standards of this development code.

- Any existing street adjoining a major subdivision or land development that is subject to improvement as part of an approved land development permit (or provides access to the subject property/permit) shall be included in any guarantee required by the Community Development Director in accordance with Article 15, Division VI, Subdivision Improvement Guarantees of this Unified Development Code.

Sec. 8-66. Improvement of Development Entrances – Residential Subdivisions.

- Residential subdivisions not exceeding 20 lots whose entrance is on an unclassified, local or minor collector city or county road shall install offset radii and 50 foot tapers as shown in standard detail.

- All other residential subdivisions shall construct a full deceleration lane to conform to standard detail.

- Residential subdivisions exceeding 200 lots may be required to install a center turn lane and longer acceleration/deceleration lanes if the public works director determines the existing city or county road warrants it.

- In cases involving rock cuts, deep fills or cuts, proximity to floodplain, etc., the requirements of pavement widening for full acceleration/deceleration lanes may be modified or waived by the public works director following accepted engineering practice.

Sec. 8-67. Improvement of Commercial/Industrial Development Entrances.

- A full deceleration lane shall be installed per Standard Detail unless determined to be unwarranted by the public works director.

- If the public works director determines that the project’s size warrants a center turn lane and longer acceleration/deceleration lanes, then such improvements will be required.

- In cases involving rock cuts, deep fills or cuts, proximity to floodplain, etc., the requirements of pavement widening for full acceleration/deceleration lanes may be modified or waived by the public works director following accepted engineering practice.

Sec. 8-68. Private Streets.

- Private streets shall only be permitted if approved by the Mayor and City Council, and in all cases shall meet all requirements and standards that apply to public streets.

- Streets and associated storm drainage serving multi-family residential development consisting of 12 or more units shall be constructed to public local residential street standards as established by this development code.

- See Secs. 15-75 and 15-76 for requirements for subdivisions involving private streets.

- All property served by a private street shall be subject to restrictive covenants running with the land that provide and acknowledge that the street is private and not maintained by the City and that maintenance of the street is the responsibility of the owners of such property or private association of the owners.

- Each private street shall be denoted as “private” on the street name sign.

- Any gate placed across a private street that limits access to a subdivision or development shall provide for unimpeded access by emergency vehicles, governmental vehicles on official business, and public and private delivery services including the U.S. Postal Service. Accessibility to such gated communities shall comply with all standards and requirements of the City for access activation, and shall be of breakaway or other construction acceptable to the City.

Sec. 8-69. Grade and Design Speed.

- All streets shall have a minimum grade of 1%. Minimum grade on cul-de-sacs should be 1.5% to maintain 1% in curb line.

- Minimum design speeds and maximum grades for proposed streets by street classification shall be as specified in Table 8-2 (maximum grade for streets without curb and gutter shall be 14%).

Table 8-2

Minimum Design Speed and Maximum Grade for Streets by Street Classification

| Street Classification | Maximum Grade | Minimum Design Speed |
| --- | --- | --- |
| Arterial | 8% | 55 mph |
| Major Collector | 10% | 45 mph |
| Minor Collector | 15% | 35 mph |
| Local | 18% | 25 mph |
| Alley | Varies | Varies |

Sec. 8-70. Cul-de-sacs and Dead-End Streets.

- A cul-de-sac shall be no more than 600 feet long unless necessitated by topographic or other conditions and approved by the community development director. Cul-de-sacs shall terminate in a circular turnaround having a minimum right-of-way of, for residential streets at least 100 feet in diameter, and a paved turnaround with a minimum outside diameter of 80 feet (see standard detail 400.03 in article 21 of this development code). On commercial and industrial streets, cul-de-sacs shall have a right-of-way diameter no less than 160 feet, and a pavement diameter of 124 feet (see standard detail 400.04 in article 21 of this…

# Article 8, Land Development Requirements.

## STREET LIGHTING

Sec. 8-76. Street Lighting.

(a) Required. Street lighting shall be required on all public and private streets and shall be

installed according to design standards of the power company servicing the subdivision or

development.

Any person or entity developing a new subdivision project constructing new streets to be dedicated to the City of Powder Springs or utilizing existing city or county roadways (or combination thereof) shall be required to provide streetlights which conform to the standards included in this section. In order to ensure adequate illumination of public rights-of-way and promote safety and security, the American National Standard Practice for Roadway Lighting of the Illuminating Engineering Society, as approved by the American National Standards Institute (1983), and as from time to time amended, is hereby adopted as the standard for the installation and operation of lighting in the city, with the following exceptions:

(1)	Lighting fixtures installed within the public rights-of-way to be operated for the purpose of street illumination shall comply with the following standards. The minimum average horizontal footcandle illumination (AHFI) level by roadway classification shall be:

| Roadway Classification | Commercial Area (AHFI) | Intermediate Area (AHFI) | Residential Area (AHFI) |
| --- | --- | --- | --- |
| Major | 1.2 | 0.9 | 0.6 |
| Collector | 0.8 | 0.6 | 0.4 |
| Local or residential | 0.6 | 0.5 | 0.3 |

The uniformity of illumination shall be such that the point of lowest illumination shall have at least one-third of the average horizontal footcandle required illumination level, except that on local or residential streets it may be not less than one-sixth of this average. The fixtures shall be mounted a minimum of 25 feet above the ground, and each fixture shall have appropriate arm length to illuminate the street.

(b) Plan. The subdivider or developer shall submit a street light layout prepared by the utility

company which will provide the lighting service showing the exact location of street

lights within the subdivision. The lighting layout must be approved by the public works director. Director of Infrastructure and Development, or his/her designee.

(c) Costs. The subdivider or developer must pay for all costs to the power company for pole

fixtures, and other related items or materials necessary for the installation. Evidence of such completed payment shall be provided to the Community Development Director at the time of final plat submittal.  The subdivider or developer shall bear the costs for all monthly street light charges until all residential structures have been completed. At such time of completion, the subdivider or developer shall provide written notification to the City of Powder Springs (via its Community Development Director) requesting that the billing be transferred to the City of Powder Springs. Any such request for transfer of billing shall be presented to and approved by the Mayor and Council.  for a full year payable

in advance at the time of final plat approval.

Article 1, General Provisions

## RENTAL CAP

Sec. 1-81. Homeowner's association.

- Required. For any residential development containing community open space, stormwater detention facilities or other lands in common ownership, a homeowner's association, which provides for building and grounds maintenance and repair, insurance and working capital shall be required. The homeowner's association must be maintained as the responsible owner of such facilities and lands and shall not be dissolved unless another ownership entity is accepted by the City of Powder Springs.

- Mandatory membership. Membership in the homeowner's association must be mandatory for each property in the development.

- Bylaws and covenants. Such associations must also include homeowner's declaration and bylaws, including covenants, conditions and regulations applicable to each property in the development. The covenants, conditions and regulations shall, at a minimum, regulate and control the following: animals, including household pets; signs exterior items such as fences, lawn ornaments, and landscape areas and buffers; building improvements; outside storage; overnight parking of vehicles; decorations and holiday lighting, trash collection containers.

- Approval. Covenants are subject to approval by the City Attorney and such approval shall be obtained prior to recording.

- Reserve account. At the time control of the association is passed from the declarant to the property owners, the association shall have a reserve account of not less than $125.00 per occupied residence. There shall be no debt incurred to the association.

- Enforcement. The declaration and bylaws shall be enforced by an association management company, which shall have the power to compel the payment of membership dues and assessments.

- Condominium association. For condominium projects, incorporation of a condominium association consistent with state law will serve in lieu of the requirements of this section.

- Owner Occupancy. Declarant or any builder constructing homes within the property must sell any such home for owner occupancy only. Thereafter, leasing of any units within the property shall not exceed five (5) percent of the total units for the entire Development. The covenants shall contain a provision that each residential unit shall be owner occupied. Leasing of any residential unit shall be limited to no more than five percent (5%) of the total number of homes and for no lease term of less than one year. The Homeowners Association (or Property Owner’s Association) must maintain records dealing with leases within the Development, and such records shall be subject to review by the City of Powder Springs personnel with regard to enforcement of this provision limiting the total number of leases at the time of such request by the city. The Association agrees to provide upon request from the City of Powder Springs any and all information relating to existing leases at the time of such request by the City. The covenants shall name the City of Powder Springs as a third party beneficiary for the purpose of enforcing the leasing provisions. No amendment to the leasing provisions shall be made without prior written approval of the City of Powder Springs.

# Article 2, Table of Permitted and Special Uses in Mixed-Use and Non-Residential Zoning Districts.

## HOME OCCUPATIONS IN CBD AND MXU

## PACKAGE STORE IN NRC

Sec. 2-22. Table of Permitted and Special Uses in Mixed-Use and Non-Residential Zoning Districts.

Table 2-3
Permitted and Special Uses in
Mixed-Use and Non-residential Zoning Districts

| Use | See also | MXU | O-I | NRC | CRC | CBD | BP | LI | HI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACCESSORY |  |  |  |  |  |  |  |  |  |
| Automated teller machine (with drive-through) | Sec. 4-35 | X | X | P | P | X | P | X | X |
| Automated teller machine (without drive-through) | Sec. 4-35 | P | P | P | P | P | P | P | P |
| Automotive Broker, Office Only (within office occupied by an established permitted use. |  | X | P | P | P | P | P | P | P |
| Business services, accessory |  | P | P | P | P | P | P | P | P |
| Caretaker's residence | Sec. 4-65 | X | X | X | X | P | P | P | P |
| Carnival or amusement park not on fairground | Sec. 4-85 | X | X | X | S | S | S | X | X |
| Carport or garage | Sec. 4-05 | P | P | X | X | P | P | P | P |
| Catering establishment |  | X | X | X | P | P | P | X | X |
| Christmas tree sales facility, temporary |  | X | X | P | P | P | P | P | X |
| Construction field office | Sec. 4-210 | P | P | P | P | P | P | P | P |
| Fence | Sec. 4-135 | See Table 4.1 |  |  |  |  |  |  |  |
| Food truck | Sec. 4-145 | P | X | P | P | P | P | P | X |
| Helicopter landing pad |  | X | X | X | S | X | S | S | S |
| Home occupation | Sec 4-170 | P | X | X | X | P | X | X | P |
| Use | See also | MXU | O-I | NRC | CRC | CBD | BP | LI | HI |
| COMMERCIAL |  |  |  |  |  |  |  |  |  |
| Open air business |  | X | X | X | X | X | S | S | P |
| Package store |  | X | X | X P | S | X | X | X | X |
| Parking lot, off-site |  | S | S | S | S | S | S | S | S |
| Parking structure |  | S | X | X | S | S | S | S | X |
| Pawn shop |  | X | X | X | X | X | X | X | X |
| Payday loan establishment |  | X | X | X | X | X | X | X | X |
| Personal service establishment, apparel |  | P | X | P | P | P | P | X | X |

SPECIFIC USE PROVISIONS – FUEL PUMPS

Sec. 4-150. Fuel Pump.

- All gasoline or other fuel pumps, where permitted, shall be located at least forty feet from the side or rear property line and at least 25 feet from any public right-of-way.

- Fuel pumps must be located at least 100 feet from any residential zoning district boundary.

- Convenience Stores with fuel pumps when permitted in the CRC zoning district must be a minimum  of 4500 sf meet the requirements below. Exceptions to these requirements may be considered as a special use by the Mayor and Council.

  - Minimum size. Convenience store associated with fuel pumps is 4500 sf.

  - Development Requirements. The convenience store with fuel pumps must be part of an overall commercial development with separate buildings for restaurant or retail uses. The convenience store with fuel pumps shall not exceed_50 % of the development site and must be constructed concurrent with the other uses.