##### LEGAL DESCRIPTION – SOUTHERN TRACT

(1.25 ACRE CUT-OUT)

All that tract or parcel of land lying and being in Land Lot 882 of the 19th District, 2nd Section, Cobb County, Georgia, and being more particularly described as follows:

BEGINNING at a point located at the intersection of the southeasterly right-of-way of Story Road (50’ right-of-way) and the easterly right-of-way of Riding Woods Drive (50’ right-of-way); having thus established the TRUE POINT OF BEGINNING leaving said point and running in a generally northeasterly direction along and following the southeasterly right-of-way of said Story Road for the following courses and distances: thence running in a northeasterly direction along a curve to the left for an arc length of 59.85 feet (said arc being subtended by a chord of N56°15’24”E – 59.84 feet and having a radius of 946.02 feet) to a point; thence running N56°40’03”E for a distance of 169.83 feet to a point; thence running in a northeasterly direction along a curve to the right for an arc length of 77.35 feet (said arc being subtended by a chord of N57°54’29”E – 77.35  feet and having a radius of 18,828.22 feet) to an iron pin set (#4 rebar w/cap) thence leaving said right-of-way and running S28°26’34”E for a distance of 273.07 feet to an iron pin set (#4 rebar w/cap); thence running N88°44’16”W for a distance of 163.03 feet to an iron pin found (#3 rebar); thence running N88°46’37”W for a distance of 225.64 feet to an iron pin found (#4 rebar) located on the east right-of-way of Riding Woods Drive; thence running N01°11’45”E along the east right-of-way of Riding Woods Drive for a distance of 64.06 feet to a point which is the TRUE POINT OF BEGINNING. Said tract contains ±1.25 acres (±54,442 sq. ft.).