Tract 1 (Estate of Lucille Garrett, Owner)

(Survey dated February 17, 2025)

All that tract or parcel of land lying and being in Land Lots 642 & 691 of the 19th District, 2nd Section, Cobb County, Georgia, and being more particularly described as follows:

TO FIND THE TRUE POINT OF BEGINNING, COMMENCE at a 3/4" open top pipe found at the corner common to Land Lots 642, 643, 690, and 691;  thence along the line common to Land Lots 642 and 691 N88°29’13”E for a distance of 44.89 feet to a 1/2" rebar found at the intersection of said Land Lot line and the westerly right of way of Ernest Barrett Parkway (a.k.a. East-West Connector, a.k.a. West Cobb Loop) (a 109 foot public right of way and limited access highway); thence traversing said right of way N87°31’24”E for a distance of 109.73 feet to a 1/2” rebar with cap set at the intersection of the easterly right of way of Ernest Barrett Parkway, and the apparent southerly right of way of Old Horseshoe Bend Road (a variable width public right of way), said point being the TRUE POINT OF BEGINNING.

FROM THE TRUE POINT OF BEGINNING AS THUS ESTABLISHED; thence leaving said easterly right of way and along said southerly right of way  N88°38'47"E for a distance of 326.92 feet to a 1/2" rebar with cap set at the intersection of the easterly right of way of Old Horseshoe Bend and the westerly right of way of Horseshoe Bend Road (a 30 foot public right of way); thence along said westerly right of way the following courses and distances:

S27°18'44"W for a distance of 208.67 feet to a point;

37.37 feet along the arc of a curve to the right, said curve having a radius of 79.14 feet and being subtended by a chord of S38°48'07"W, 37.03 feet to a point;

S52°19'51"W for a distance of 68.30 feet to a point;

106.71 feet along the arc of a curve to the right, said curve having a radius of 242.07 feet and being subtended by a chord of S68°18’13”W, 105.85 feet to a 1/2" rebar with cap set at the intersection of said westerly right of way and the easterly right of way of Ernest Barrett Parkway; thence along said easterly right of way the following courses and distances:

N09°57'52"W for a distance of 42.40 feet to a concrete monument found;

250.33 feet along the arc of a curve to the left, said curve having a radius of 6929.99 feet and being subtended by a chord of N11°05’08”W, 250.32 feet to a 1/2" rebar with cap set at the intersection of the easterly right of way of Ernest Barrett Parkway and the apparent southerly right of way of Old Horseshoe Bend Road, said point being the TRUE POINT OF BEGINNING.

Said tract or parcel of land contains 1.392 acres.