##### LEGAL DESCRIPTION – NORTHERN TRACT

(2.50 ACRE CUT-OUT)

All that tract or parcel of land lying and being in Land Lots 882 and 883 of the 19th District, 2nd Section, Cobb County, Georgia, and being more particularly described as follows:

BEGINNING at an iron pin set (#4 rebar w/cap) located on the northwesterly right-of-way of Story Road (50’ right-of-way) said iron pin being located 375.79 feet from the intersection of the said northwesterly right-of-way of Story Road and the northeasterly right-of-way of Hiram Lithia Springs Road (50’ right-of-way) as measured in a northeasterly direction along the said northwesterly right-of-way of Story Road; having thus established the TRUE POINT OF BEGINNING leaving said point and right-of-way and running N00°04’49”E for a distance of 150.61 feet to an iron pin set (#4 rebar w/cap); thence running N00°02’19”W for a distance of 76.52 feet to an iron pin set (#4 rebar w/cap); thence running N89°57’41”E for a distance of 234.56 feet to an iron pin set (#4 rebar w/cap); thence running N23°25’45”E for a distance of 93.35 feet to an iron pin set (#4 rebar w/cap); thence running N89°57’41”E for a distance of 477.67 feet to an iron pin set located on the northwesterly right-of-way of said Story Road; thence running in a generally southwesterly direction along and following the northwesterly right-of-way of said Story Road for the following courses and distances: running thence S56°40’03”W for a distance of 46.24 feet to a point; thence running in a southwesterly direction along a curve to the right for an arc length of 237.07 feet (said arc being subtended by a chord of S61°57’46”W – 236.38 feet to a point and having a radius of 896.02 feet) to a point; thence running S70°26’51”W for a distance of 98.34 feet to a point; thence running in a southwesterly direction along a curve to the left for an arc length of 322.52 feet (said arc being subtended by a chord of S71°03’25”W – 322.50 feet and having a radius of 7,348.08 feet) to a point; thence running S69°28’26”W for a distance of 111.62 feet to an iron pin set (#4 rebar) which is the TRUE POINT OF BEGINNING. Said tract contains ±2.50 acres (±108,858 sq. ft.)